WhiteWater Express Car Wash

Lockout/Tagout (LOTO) Program

December 2025

Version 1.4

Table of Contents

- Purpose & Scope

- Definitions

- Roles and responsibilities

- When Lock out Tag out is required

- Types of Hazardous Energy at Whitewater

- General Lock Out Tag Out Procedure

- Working under Lock Out Tag Out

- Release from Lock out Tag Out

- Training requirements

- Periodic Inspections and Audits

- Recordkeeping Requirements

- Machine Specific Lock out Tag out Procedure

- Appendix A

1. Purpose & Scope

The purpose of the WhiteWater Express Car Wash Lockout/Tagout (LOTO) Program is to protect all employees from injuries caused by the unexpected energization, startup, or release of stored energy during the servicing or maintenance of equipment. This program meets the requirements of OSHA 29 CFR 1910.147 – Control of Hazardous Energy.

This program applies to all Whitewater employees, contractors, and authorized personnel who service, maintain, clean, troubleshoot, or otherwise interact with equipment that requires de-energization.

This program only covers work performed on equipment that has been completely de-energized and verified as safe.

Important Note: If any task requires exposure to live electrical components or energized parts, this LOTO program does NOT apply. Such tasks require a separate Energized Electrical Work Procedure, advanced NFPA 70E training, and approval from management. Whitewater employees are not PERMITTED to perform energized electrical work.

2. Definitions

Authorized Employee: An employee trained and approved by Whitewater to apply locks/tags and perform LOTO.

Affected Employee: An employee who operates or works near equipment being serviced under LOTO but does not apply locks.

Energy-Isolating Device: A mechanical device that physically prevents the transmission of energy (e.g., MCC disconnects, breakers, valves).

De-Energized: Equipment has been shut off, isolated, locked out, and verified to be free of hazardous energy.

Stored Energy: Residual energy such as pressurized water, gravity, tension, heat, or rotating parts that must be relieved before work begins.

Zero-Energy State: A condition where all energy sources have been fully isolated, locked out, and verified.

3. Roles & Responsibilities

Management

- Ensure LOTO procedures meet OSHA requirements.

- Designate competent Maintenance personnel to conduct practical training and skills verification for Operational employees.

- Ensure authorized employees receive both instructional (video/classroom) and practical (hands-on) training.

- Ensure adequate locks, tags, and LOTO devices are available.

- Maintain annual audits and documentation.

- *Ensure no employee performs energized electrical work.

Supervisors

- Enforce compliance with this program during all maintenance activities.

- Maintain the equipment list requiring LOTO.

- Verify employees performing LOTO are authorized.

- Ensure MCC areas remain secured and accessible only to trained individuals.

Authorized Employees

- Perform LOTO exactly as outlined.

- Use only company-issued LOTO devices.

- Verify zero-energy state before work begins.

- Maintain control of their assigned LOTO keys at all times.

Affected Employees

- Must not attempt to start or operate locked-out equipment.

- Must follow instructions and remain clear about maintenance activities.

4. When LOTO Is Required

LOTO must be performed whenever:

- Equipment is being cleaned, serviced, maintained, inspected, or repaired.

- An employee must place any part of their body inside a machine or near moving parts.

- Guards or safety devices are removed.

- Work occurs inside the conveyor pit, around pumps, near MCC panels, or around any motor-driven equipment.

LOTO is required even if equipment is believed to be “off.”

5. Types of Hazardous Energy at Whitewater

Whitewater equipment may involve:

- Electrical Energy: 3-phase motors, MCC panels, control circuits.

- Mechanical Energy: Rotating shafts, rollers, conveyors.

- Hydraulic / Pneumatic Energy: Pressure in hoses, cylinders.

- Water Pressure: High PSI wash pumps.

- Gravity: Elevated components that may fall or shift.

- All must be controlled during LOTO.

6. General Lockout/Tagout Procedure

This is the core procedure used for most equipment at Whitewater.

Step 1 – Preparation

- Identify all energy sources that power the equipment.

- Review machine-specific procedures if available.

- Notify all affected employees.

Step 2 – Shutdown

- Power down the equipment using normal operating controls.

Step 3 – Isolation

- Turn off all energy-isolating devices (breakers, valves, switches).

- All electrical equipment must be locked out at the MCC, consistent with Whitewater’s Safety Manual.

Step 4 – Application of LOTO Devices

- Apply a lock and tag to each isolating device.

- Each authorized employee must use their own lock.

- Tags must be legible and filled out completely.

- Keys must remain in the possession of the employee.

Step 5 – Stored Energy Dissipation

- Release or block any stored mechanical, pneumatic, hydraulic, or water pressure.

- Verify moving parts have stopped.

Step 6 – Verification (Attempt Start)

- Attempt to start the machine using the normal controls.

- Confirm that the machine does not power on.

- Return controls to the "off" position.

This verification step is required before any work begins.

7. Working Under LOTO Conditions

- Only authorized employees may work on the equipment.

- No one may remove another employee’s lock.

- Maintain clear communication while work is ongoing.

8. Release From Lockout/Tagout

When work is complete:

1. Inspect the work area for tools and materials.

2. Ensure guards, panels, and safety devices are reinstalled.

3. Ensure all employees are clear of the equipment.

4. Remove locks and tags (each employee removes their own).

5. Re-energize equipment.

6. Notify affected employees that work is complete.

9. Training Requirements

Whitewater requires:

- Authorized Employee Training: Full LOTO training covering hazards, procedures, and verification.

  - Instructional: Review of the LOTO policy, energy sources, and safety videos.

  - Practical Verification: A designated Maintenance employee must verify the trainee’s ability to perform LOTO on specific equipment. This includes identifying energy sources, applying locks/tags, and verifying isolation (zero energy).

  - Documentation: Training is not complete until the "LOTO Authorization Form" is signed by both the trainee and the Maintenance trainer.

- Affected Employee Training: Awareness-level understanding of LOTO.

- Retraining: Required when job duties change, procedures change, or performance deficiencies are observed.

Training records will be maintained for employment duration + 3 years, consistent with the Electrical Safety Program.

10. Periodic Inspections and Audits

- Annual inspection of each LOTO operation to ensure compliance.

- Audit records must be kept for **a minimum of one year**.

- Corrective actions will be documented and implemented.

11. Recordkeeping Requirements

Whitewater will maintain:

- Training records

- LOTO logs

- Annual audit documentation

- Machine-specific procedures

Retention requirements match the Electrical Safety Program.

12. Machine-Specific LOTO Procedures

The following appendices include LOTO procedures for specific equipment at Whitewater. Additional procedures will be added as identified.

Appendix A – Sonnys Conveyor Machine Specific LOTO

### WhiteWater Express Car Wash

### Machine-Specific LOTO Procedure - Sonny’s Chain Conveyor

Equipment Name: Sonny’s Chain Conveyor
Location: 714-Ionia
Primary Energy Sources:

- Electrical (480V, from Motor Control Center powered by Main Distribution Panel)

- Hydraulic (motor driven by electrical power)

- Pneumatic (air tensioning system powered by air compressor)

#### 1. Purpose

To ensure the complete isolation and de-energization of all hazardous energy sources before performing pit cleaning, unjamming rollers, or any conveyor or pit maintenance.

This process must be completed by an authorized employee any time the conveyor or the trench are being serviced.

#### 2. Authorized Employees

Only trained and authorized employees may perform this procedure.

#### 3. Energy Sources and Isolation Points

| Energy Type | Description | Isolation Method |
| --- | --- | --- |
| Electrical | Power to the Motor Control Center, feeding conveyor motor, and air compressor | Turn off and lock out the Main Distribution Panel breaker supplying power to the Motor Control Center |
| Hydraulic | Conveyor hydraulic motor powered through the Motor Control Center | Turning off and locking out the Main Distribution Panel breaker de-energizes the hydraulic system |
| Pneumatic | Air tensioning device connected to air compressor | Turning off and locking out the Main Distribution Panel breaker de-energizes the air compressor. Slowly bleed residual air pressure via the release valve on the compressor. Verify by checking the pressure gauge |

#### 4. Lockout/Tagout Steps

- Notify affected employees that the conveyor will be shut down for maintenance or cleaning.

- Shut Down Equipment – Stop conveyor operation using normal controls.

- Isolate Energy Sources:

  - Turn off and Lockout/Tagout the Main Distribution Panel breaker feeding the Motor Control Center (this de-energizes conveyor electrical, hydraulic, and pneumatic systems).

  - Depressurize the pneumatic system by slowly opening the release valve to release all stored air. Verify by checking the pressure gauge.

- Verify De-energization:

  - Attempt to start the conveyor using the normal start controls to confirm it does not operate.

  - Visually confirm that the hydraulic and pneumatic systems have stopped movement and pressure has been released.

- Perform Maintenance or Cleaning Work.

- Re-energization:

  - Inspect the area, ensure all tools and personnel are clear.

  - Remove locks/tags by the person who applied them.

  - Restore power by turning on the Main Distribution Panel breaker and restarting systems per normal operations.

  - Verify that serviced equipment is functioning properly.

  - Notify all affected employees that the machine is ready for operation.

Appendix B - LOTO Authorization Form

# Authorized Employee Training and Certification Record

## Control of Hazardous Energy (Lockout/Tagout) - OSHA Standard 29 CFR 1910.147

Employee Name: _________________________________________

Department: ___________________________________________

Date of Initial Training/Authorization: __________________________

## 1. Purpose of Authorization

This document certifies that the named employee has been trained, demonstrated proficiency, and is officially authorized by the employer to perform Lockout/Tagout (LOTO) procedures for the control of hazardous energy sources, as defined by the Occupational Safety and Health Administration (OSHA).

## 2. Key Definitions

| Term | Definition and Responsibility |
| --- | --- |
| Authorized Employee | The person who locks or tags out machines or equipment to perform servicing or maintenance on that machine or equipment. This employee is primarily responsible for ensuring the machine is de-energized and zero energy state is achieved before work begins. |
| Affected Employee | An employee whose job requires them to operate a machine or equipment on which servicing or maintenance is being performed, or whose job is performed in an area where LOTO is being utilized. Affected employees are not permitted to apply or remove LOTO devices, but must respect them. |
| Lockout | The placement of a lock or physical device on an energy-isolating device, ensuring that the machine or equipment cannot be operated until the LOTO device is removed. |
| Tagout | The placement of a prominent warning tag on an energy-isolating device, indicating that the equipment must not be operated. This provides warning only and does not provide the physical restraint of a lock. |

## 3. Initial Training and Responsibilities of an Authorized Employee

The Authorized Employee must be proficient in the following aspects and will be held responsible for executing these procedures correctly:

### A. Recognition of Hazardous Energy Sources

The Authorized Employee must be able to recognize all types of hazardous energy present in the workplace and the methods required for their control.

- Types of Energy: Electrical (stored and residual), Mechanical (kinetic and potential, springs, gravity), Hydraulic, Pneumatic, Chemical, Thermal, and others.

- Energy Magnitude: Understanding the severity of the energy present.

### B. Energy Control Program Procedures (LOTO Steps)

The Authorized Employee must know, understand, and apply the company-specific LOTO procedure for each piece of equipment being serviced.

| Step | Action Required |
| --- | --- |
| Preparation | Notify all affected employees and co-workers of the intent to shut down the equipment. |
| Shutdown | Shut down the machine or equipment using the established procedure. |
| Isolation | Isolate the equipment from all energy sources (e.g., open breakers, close valves, block moving parts). |
| Application | Affix a personal lock and tag to the isolating device(s). Each Authorized Employee involved must affix their own device. |
| Stored Energy | Bleed, dissipate, restrain, or otherwise render safe all stored or residual energy. |
| Verification | Attempt to start the equipment or operate the isolating controls to verify that the isolation is successful (Trial Startup). Return controls to neutral/off position after verification. |
| Maintenance/Servicing | Perform the necessary work. |
| Restoration | Inspect the equipment area, ensure non-essential items are removed, and all guards are re-installed. Remove LOTO devices, notify affected employees, and re-energize the equipment. |

### C. Device Application and Removal

- Application: Only the Authorized Employee is permitted to apply their personal LOTO device.

- Removal: Only the Authorized Employee who applied the device is permitted to remove it.

- Exception (Procedure Required): In rare cases where the Authorized Employee is unavailable, the device may be removed by a designated supervisor ONLY after following strict company procedures, including full verification that the employee is not on the premises and notifying the employee before they return to work.

## 4. Periodic Inspection Requirements (Annual Audit)

A periodic inspection of the energy control procedure will be conducted annually to ensure that the procedure and the standard's requirements are being followed. The inspection must be performed by an Authorized Employee other than the one(s) utilizing the energy control procedure being inspected. The inspection will entail observing the authorized employee(s) actually performing the LOTO procedure on the specific machine or equipment.

## 5. Employee Acknowledgement and Certification

I hereby affirm that I have received training on the proper procedures for the Control of Hazardous Energy (Lockout/Tagout), including the recognition of energy sources, the purpose of LOTO, and the safe application and removal of energy control devices.

I understand that as an Authorized Employee, I am responsible for strictly adhering to the established company Lockout/Tagout procedures to protect myself and others from the unexpected startup or release of stored energy during servicing or maintenance. I understand that failure to follow these procedures may result in disciplinary action up to and including termination, as well as severe injury or death.

Employee Signature: ___________________________________________

Date: _______________________________________________________

## 6. Initial Certification

I hereby certify that the employee named above has received the required Initial Training in accordance with OSHA Standard 29 CFR 1910.147 and has demonstrated the necessary knowledge and skills to be designated as an Authorized Employee.

Trainer/Supervisor Name (Printed): __________________________________

Trainer/Supervisor Signature: _______________________________________

Date: ___________________________________________________________

Appendix C - Periodic LOTO Inspection Form

# WhiteWater Express Car Wash - LOTO Hands-On Evaluation Checklist

Employee Name: __________________________ Date: __________________________

Trainer: _________________________________

Equipment LOTO Completed on: __________________________________________

| Step | Completed (Y/N) | Trainer Initials |
| --- | --- | --- |
| 1. Properly notifies affected employees before shutdown. | _____________________ | ______________________ |
| 2. Correctly identifies the equipment requiring lockout. | _____________________ | ______________________ |
| 3. Shuts down equipment using normal procedures. | _____________________ | ______________________ |
| 4. Isolates all applicable energy sources (electrical, pneumatic, hydraulic, etc.). | _____________________ | ______________________ |
| 5. Applies personal lock and tag securely to each isolation point. | _____________________ | ______________________ |
| 6. Releases any stored energy (bleeds pressure, discharges capacitors, etc.). | _____________________ | ______________________ |
| 7. Verifies zero energy state by attempting to start equipment safely. | _____________________ | ______________________ |
| 8. Performs maintenance tasks only after confirming energy isolation. | _____________________ | ______________________ |
| 9. Removes tools and restores work area before removing lock/tag. | _____________________ | ______________________ |
| 10. Removes personal lock and tag correctly (only their own lock). | ________________________ | _____________________ |
| 11. Restarts equipment safely and notifies affected employees. | ________________________ | _____________________ |

Trainer Signature: ___________________________   Date: _______________